

INTRODUCTION

( Preparations for entries	2

( Error warning	2

( Sample receipt	2



ENTRIES	3

1. Cashier code entries	3

2. Item entries	4

(1) Single Item Entries	4

(2) Repeat Entries	4

(3) Multiplication entries	5

3. Display of subtotals	6

4. Finalization of transaction	6

(1) Cash, Cheque,Visa/Master Card/Amex and Debit Card 	6

5. Automatic tax	7

6. PST tax exempt and Band Number Enteries	7

7. Discount/Coupon entries	8

(1) Item Discount	8

(2) Gold Crown Coupon	8

8. No sale 	9

9. Refund entries	9

10. Float  entries	9

11. U.S. Currency Conversion	10



CORRECTIONS	11

1.  Correction of the last entry (error correct)	11

2.  Correction of past item or earlier item entries (void)	11

3.  Correction of all the sale entries (trans void)	11



Resetting (Z) Reports using the auto report key	12



Mode Switch Positions and Mode Keys	13



�INTRODUCTION

�

( Preparations for entries



(1) Put the operator key in the mode switch and turn it to the REG position.

(2) Check to see if your register has both the journal and receipt rolls.  If your register lacks these rolls or 

     has low rolls, install new paper rolls or replace the old rolls with new ones.

(3) Sign on a Cashier.  See "Cashier Code Entries" page 3.



( Error warning



�In the following examples, your register will go into an error state accompanied with a warning beep and the error message “MISOPERATION” or “ENTRY ERROR” on the display.  Clear the error state by 

pressing the           key and perform the correct operation.



(1) When you enter an over 32-digit number (entry limit overflow):

      ( Cancel the entry and re-enter a correct number.



(2) When you make an error in key operation:

      ( Clear the error and operate the keys correctly.



(3) When you make an entry beyond a programmed amount entry limit:

      ( Check to see if the entered amount is correct.  If it is correct, it can be rung up in the MGR

         mode.  Contact your manager.



������(4) When an including-tax subtotal exceeds eight digits:

�      ( Delete the subtotal by pressing the            key and press the          ,         ,          ,         ,          ,            	

�	or          key to finalize the transaction.



( Sample receipt



�

��	                               Date/Time                                                             Server #

�	Register no./Consecutive no.	

		

�                                                                                                           Item entry

		

�		Merchandise Subtotal



�		PST Tax

�		GST Tax



�		Sales total

�		Cash amount tendered

�		Change due

			�ENTRIES

�

1. Cashier code entries



Prior to “Start of Day”, the Cashier is required to enter his or her assigned server code.  



Procedure

�

��To enter a Cashier code:	                        	        #			

	#= Cashier # 1,2,3, or 4



Note:	(  A Cashier is signed off when a different server signs on.

	(  Once an item entry has been made the Cashier number cannot be changed.

	(  A Cashier is signed off when a Day-End Z Report is taken.



�2. Item entries

(1) Single item entries

( Entries into departments

 

Procedure



�

��	   Enter Price  			      (Direct Dept. entries)

	 (max. 7 digits)





Example:  Selling a $10.00 item (E’DAY CARDS) and a $5.00 item (SHOEBOX) for cash



	   Key operation					   Print

���

�		1000  

�                            

                            500

                   

�



�



(2) Repeat entries

You can use this function for entering a sale of two or more same items.



Example:  Selling these items for cash



���		Dept./PLU no.			Unit price		Q’ty



	    E’DAY CARDS			   $2.00			   3

                 SHOEBOX				   $4.00			   2

	Key operation					                Print

�����

�Repeated	   200

��dept.

�entry

�

���

Repeated	   450

��dept.

entry

��



�

















 (3) Multiplication entries

Use this feature when you need to enter two or more same items.

This feature helps when you sell a large quantity of items or need to enter quantities that contain decimals.



Procedure

		



����

���   Q’ty                                      Unit price                                     --------------------------------- Dept. entry







(  Q’ty:  Up to four-digit integer 

(  Unit price:  Less than the programmed HALO

(  Q’ty x unit price:  Up to seven digits



Example:  Selling these items for cash



		Dept./PLU no.			Unit price		  Q’ty

���

	     E’DAY CARDS			 $2.00			  3

                  SHOEBOX				 $4.50			  2





	Key operation					                Print

����

�	                 3 

�                            200 

�                               2

 	                450

�

�





�3. Display of subtotals



Your register provides these two types of subtotals:

�(1) Merchandise subtotal

Press the         key at any point during a transaction.  The net sale subtotal - excluding add-on tax - and “MDSE ST” will appear in the display.

�(2) Including-tax subtotal (full subtotal)

Press the         key at any point during a transaction.  The sale subtotal including tax and “SUBTOTAL” will appear in the display and the “ST” lamp will light up.



4. Finalization of transaction

�(1) Cash, Visa/Master Card, Debit Card or Gift Cert tendering

�������Press the         key to get an including-tax subtotal, enter the amount tendered by your customer, then 

press the            ,            ,            ,            ,            ,            or           

When the amount tendered is greater than the amount of the sale, your register will show the change due amount (Only Allowed for CASH, CHEQ and DEBIT Tender), the message "CHANGE" will appear in the display, and the “CG” lamp will light up.  Otherwise your register will show a deficit and the message “DUE”.  Make the correct tender entry.



Example:  Your customer pays $10.00 for an including-tax subtotal of $7.35.

( Cash tendering





	Key operation					                 Print

��        	 	 

�			~

�

�

		1000





�

5. Automatic tax

Your register is programmed with the following taxes: Tax1 is PST Tax and Tax4 is GST Tax.  The register will automatically compute the tax on any item that is set to be taxable under the taxes listed above.



Example:  Selling three $2.00 items (E’DAY CARDS, TAX1 & TAX4) and one $9.00 item (SHOEBOX, TAX1 & TAX4) for cash.



	Key operation					                 Print

����        	 	 

�		      3

�		   200

		   900

�			

�







6. PST Tax Exempt and Band Number enteries

�Your register is programmed with the ability to exempt Tax1 (PST Tax).  The register will automatically cancel Tax1 when the            key is depress after a subtotal. You must enter the Band number of the person(s) requiring this operation.



Example:  Selling three $2.00 items (E’DAY CARDS, PST and GST, PST Exempt)  for cash using Band number 12345.



	Key operation					                 Print

����        	 	 

�		      3

		   200

		   �		

	

�                         12345

�



�



�7. Discount/Coupon entries

��For item discount tendering, use the          key.

Enter the sale item then enter the discount rate and depress the           key immediately after entering sale item. 

�

For coupon tendering, use the            key.

��

Enter all sale items then depress the             key followed by the           key.

(1) Discount tendering for the sale item

Example:  Selling E’DAY CARDS with 10% tem Discount; 

	

	Key operation					                 Print

��	 	 

��	     	  500        	

�		    10

�		    

�

                            



















(2) Gold Crown Coupon Tendering for the Subtotal

Example:  Selling for cash an $8.25 item of E’DAY CARDS





	Key operation					                 Print

���	 	 

��	                825	

               		

�	           �	     25 



��8. No sale 

��Simply press the          key without any entry and the drawer will open and the printer will print “NO SALE” on both the journal and the receipt.  If your machine is programmed to allow you to print a non-add code number before pressing the         key, a no-sale entry can be achieved with a non-add code number printed. 





	Key operation					                 Print

��	 	 

��	



                        



9. Refunds (Requires Manager Mode)

�

�If a refund item is to be entered into a department, turn the Mode Lock to the MGR position, enter the amount of the refund, then press the         key followed by the corresponding department key.

 

Example:  These items sold for cash are returned.

	( One $1.00 item of E’DAY CARDS

	( Seven $2.00 items of SHOEBOX



	Key operation					                 Print

����	 	 

��	       100     	

             	       7	

��	        200    

�                        

�





10. Floats

�

Entering of the FLOAT is required for the start of the day.

 

Example: Entering of a $50.00 Float



��

�                  5000   







11. U.S. Currency conversion

�Your register allows payment entries of U.S. foreign currency.

Pressing the                key creates a subtotal in foreign currency.



Only cash can be handled after currency conversion.

Procedure

��

��

���

������











�

��  







* Rate:  0.0000 to 9999.9999



Example: Selling $10.00 item (E’DAY CARDS, PST and GST)  for cash and tendering U.S. cash at a U.S Exchange rate of .725  ($1.00 US is worth $1.379 CDN or $1.00 CDN is worth $0.725 US)







	Key operation					                 Print

���	 	 

�	             1000    	

�



�

�             	���	

���

�                      













�CORRECTIONS

1.  Correction of the last entry (Error Correction)



If you make an entry error relating to a Department entry, discount or coupon, you can void this entry by pressing the "ERR CORR" key immediately after the incorrect entry.



Example:



	Key operation					                 Print

����	 	 

    	         300       

��	                 

��















2.  Correction of past item or earlier item entries ( Void)



With this function, you can void any incorrect positive entries made during a transaction if you find the mistake before finalizing the transaction (i.e.  pressing the CA/AT key).  This function is applicable to Department entries only. Enter the item amount, depress the “ VOID” key then depress the department key that you wish to void.

Example:

	Key operation					                 Print

����	 	 

�	          100        

	          200       

��            100

�



�





3.  Correction of all sale items (TRANS Void)

With this function, you can void all of the sale that has been entered at the cash register.  After depressing the “SBTL” key, depress the “TRANS VOID” key. This will cancel the entire sale that had been entered.



�Resetting (Z) of Sales Totals



-Use the resetting function (Z) when you need to clear the register’s memory.  Resetting prints all sales information and clears the entire memory except for the GT1 thru GT3, reset count, and consecutive number.

�

To perform your Daily Reports, turn the Mode key to the X1/Z1 Position and depress the                key.

Your Daily Sales will be issued:	-General Report (Read)

				-Group Report (Read)

				-General Report (Reset)*



* Also resets Group Report totals



�



MODE SWITCH POSITIONS AND MODE KEYS



The mode switch can be operated by inserting one of the mode keys - manager (MA), submanager (SM), and operator (OP) keys.  These keys can be inserted or removed only when the switch is in the “REG” or “OFF” position.

�



The mode switch has these settings:



OFF:		The OFF mode locks all register operations.  No change occurs to register data.

OP X/Z:		This setting allows cashiers to take X or Z reports for their sales information.  It can also be 		used for displaying the time and printing the employee arrival and departure times.

REG:		For entering sales

PGM1:		Program Mode

PGM2:		Program Mode

MGR:		For functions that require Manager Mode (eg. Refunds)

X1/Z1:		To take daily reports

X2/Z2:		To take periodic reports

Mid-Range System Solution





Mid-Range System Solution



�	  � PAGE �2�                                 � SHARP ELECTRONICS OF CANADA LTD.	                                 





�� SHARP ELECTRONICS OF CANADA LTD.	                            � PAGE �3�                     







C L





DEBIT



AMEX



MSTR

CARD



CA/AT



VISA



CHEQ



C L





DISCVMOND



08/27/96  6:01PM  01

0001#1098      



         

E’DAY CARDS    	$10.00

SHOEBOX       	 $5.00



MDSE ST        $15.00

PST             $1.20  

GST             $1.05	       

***TOTAL   	 $15.00

Cash          	$20.00 

Change        	 $5.00









CASHR#







DEPT



E’DAY

CARDS





E’DAY CARDS    	$10.00

SHOEBOX       	 $5.00

MDSE ST        $15.00

PST             $1.20  

GST             $1.05	       

CASH         $17.25 



SHOE

BOX



SUB

TOTAL





CASH



E’DAY

CARDS











E’DAY CARDS     $2.00

E’DAY CARDS     $2.00

E’DAY CARDS     $2.00

SHOEBOX	    $4.50

SHOEBOX	    $4.50

MDSE ST        $15.00

PST 8%          $1.20  

GST 7%          $1.05	       

CASH         $17.25 



 



E’DAY

CARDS









E’DAY

CARDS









SHOE

BOX









SHOE

BOX









SUB

TOTL



CA/

AT





DEPT



@/

FOR





              3@ 2.00

 E’DAY CARDS     $6.00

              2@ 4.50

 SHOEBOX         $9.00

 MDSE ST        $15.00

 PST             $1.20  

 GST             $1.05	       

CASH         $17.25 



@/

FOR



E’DAY

CARD









@/

FOR



SHOE

BOX









SUB

TOTL



CA/

AT



MDSE

SBTL













SBTL













SBTL













DISCVMOND



DEBIT



AMEX



MSTR

CARD



VISA



CHEQ



CA/AT



 

 ***TOTAL       	$7.35

 CASH          $10.00

  CHANGE         $2.65



SBTL













CA/

AT





	3@ 2.00

 E’DAY CARDS     $6.00	

 SHOEBOX	     $9.00

 MDSE ST        $15.00

 PST             $1.20  

 GST             $1.05	       

CASH         $17.25 



@/

FOR



E’DAY

CARD



















SHOE

BOX



















SUB

TOTL



CA/

AT



PST

EXMP



@/

FOR





	3@ 2.00

 E’DAY CARDS     $6.00

 #12345

 PST ST          $0.00   

 MDSE ST         $6.00

 GST             $0.42	       

 CASH         $ 6.42 



E’DAY

CARD



















SUB

TOTL



BAND

#



PST

EXMP



CA/

AT



ITEM DISC



(1

















ITEM DISC



(1

















GC

COUP



gc

coup











SBTL





                    

 E’DAY CARDS     $5.00

 

                 -10%

 I-DISC         $ 0.50

 MDSE ST         $4.50

 PST             $0.36  

 GST             $0.32



 CASH         $  5.18

  



E’DAY

CARD



















ITEM

DISC





(1

















SBTL        



(1

















CA/

AT



E’DAY

CARD

















 E’DAY CARDS 	$8.25

 GC COUP  	-0.25

 MDSE ST         $8.00

 PST             $0.64  

 GST             $0.56



 CASH         $  9.20

  

***TOTAL       	$7.75

 CASH          $ 8.00

  CHANGE        $ 0.25



  

 



SBTL



















GC

COUP



















SUB

TOTL



CA/

AT



NS



















NS



















NS





  NO SALE



 

 







RFND











RFND











E’DAY

CARD





















 E’DAY CARDS   R-$1.00

              -7@ 2.00

 SHOEBOX      R-$14.00

 MDSE ST       -$15.00

 PST            -$1.20  

 GST            -$1.05	       

CHANGE        $17.25 

 	



@/

FOR

















RFND











SHOE

BOX



















SUB

TOTL



CA/

AT









CA/AT



FLOAT



U.S.

EXCHANGE



* Enter Exchange Rate



Amount

tendered

(max. 8 digits)



U.S.

EXCHANGE



* Rate



 CA/

     AT



After an entry is completed 



Next registration or payment in domestic currency



CL



E’DAY

CARDS





 E’DAY CARDS  	 $10.00

 MDSE ST        $10.00

 PST            $0.80

 GST            $0.70



**TOTAL     	 $11.50

 US EXCHG       0.725

                  8.34 

 CASH            10.00

 CHANGE		$2.29



 

 



SUB

TOTAL



U.S.

EXCHANGE



              Domestic currency-----

                   Conversion rate-----

           Foreign currency Total---------------------------------      

   Foreign currency Tendered----      

   Change Domestic currency----







725



(



















10000





CA/AT



Amount tendered in foreign currency



E’DAY

CARD





 E’DAY CARDS	$3.00

 E’DAY CARDS 	V-3.00

 

 CASH	$0.00



ERR

CORR







CA/

  AT













 E’DAY CARDS	$1.00

 SHOEBOX	$2.00

 E’DAY CARDS   V-$1.00

 MDSE ST         $2.00

 PST             $0.16  

 GST             $0.14



 CASH         $  2.30



E’DAY

CARD













SHOE

BOX













E’DAY

CARD













VOID











SUB

TOTL



CA/

  AT









AUTO

REPORT








